Optimal Reservoir Operations to Maximize Profit During Irrigation Season

Ammon Wallace

CEE 5410 – Dr. Rosenberg

09/29/2026

Introduction

During the irrigation season, prices increase as the season gets closer to the end. The reservoir has a finite storage, meaning the irrigator can’t wait until the last minute to distribute all the water. The report explains how to operate the reservoir to maximize profit. This report gives the model formulation, the optimal solution, and a model analysis.

Model Formulation

The dimension of the model are the months (June, July, and August) and location of water (Spillway, Diversion, and Storage). The decision variables are how much water should go to each location during each month. These variables are expressed as X(location, month) in units of acre-ft. This notation means there are nine decision variables.

The objective function maximizes profit based on the irrigation benefits (Table 1). The irrigations benefits are expressed as c(m). The objective function can be expressed as MaxZ=c(mi)*X(Diversion ,mi), [$]. The other decision variables are only not included in the objective function because they do not contribute any profit.

The constraints are listed with the corresponding mathematical equation and units: The ending storage is greater than or equal to the beginning storage, XStorage, Aug>=3000, [ac-ft]. The max capacity of the reservoir is 9000 ac-ft, XStorage,mi<=9000, [ac-ft]. The reservoir must follow the mass balance equation. XStor.,mi-XStor.,mi-1=inflowi-XDiv.,mi-XSpill,mi, [ac-ft]. (Note: inflows are shown in Table 2)

Solution

GAMS (GAMS Development Corporation, 2026) found the optimal solution that provides the maximum profit. Table 3 contains the optimal diversion schedule. The maximum profit, given by the objective function, is $3,774,000. Based on the GAMS solution the constraints that binds the objective function are the mass balance constraint during all months, the ending storage being greater than or equal to initial storage in August, and the reservoir capacity in July. The rest of the constraints are slack during the respective months.

Analysis

The diversion schedule is optimal because the most water diverted is in the last month and none of the water goes on the spillway to the river. All water contributes the most profit possible. This problem helped grow my understanding on model formulation; specifically, how to mathematically express the model. This model does not consider the demand of farmers which might be a concern for the company manager.

Conclusion

In conclusion by following the optimal diversion schedule the profit will be $3,774,000. This is the maximum profit based on the constraints and information given. GAMS programing and modeling software was used to find the solution to the problem.

Appendix A – Tables

Table 1: Irrigation Benefits (Rosenberg, 2026)

| Month | Irrigation Benefits ($/acre-foot) |
| --- | --- |
| June | 150 |
| July | 170 |
| August | 425 |

Table 2: Inflows (Rosenberg, 2026)

| Month | Inflows (acre-feet) |
| --- | --- |
| June | 5,000 |
| July | 3,200 |
| August | 2,000 |

Table 3: Diversion Schedule

| Month | Diversion Amount (acre-feet) | Corresponding Reservoir Level (acre-feet) |
| --- | --- | --- |
| June | 0 | 8,000 |
| July | 2,200 | 9,000 |
| August | 8,000 | 3,000 |

Appendix B - References

GAMS Development Corporation. (2026). General Algebraic Modeling System (GAMS) (Release 54.4.0) [Computer software]. https://www.gams.com/

Rosenberg, D. (2026). HW3-LinearPrograminGAMS. Logan; Utah State University.

AI Statement

I used AI (Gemini) to help me fix some errors in my GAMS. I was having a hard time using the $ operator in GAMS and I couldn’t figure out the syntax. AI helped me understand how the syntax works. This follows the AI policy because I did not copy and paste anything from AI into my GAMS. I did not let AI govern any decision I made in my model. Here is the AI chat: https://share.gemini.google/kPXsfVZFl82a